Upgrade tool for manual

- Run the program --“UpdateTool.exe”

（If the operating system of your computer is WIN 7 32bit or 64bit, Please run the “UpdateTool.exe” with administrator privileges）

Interface of the tool：

2. Before beginning the upgrade, please check the port is 9025 or 9026.

3、Normal upgrade

（PS：Please make sure the path of software without Chinese）

Click the “START” button. Tool will backup the QCN files at first, The file will be saved in the folder SW.

Update modem

Update AP

Restore QCN，

Prompt box will pop up when the upgrade is complete.

Click the “STOP” button and Click ”YES”

4. Repair Mode

（1）To update modem failed

Holding down the up-key and down-key of volume, then connect PC. Now the port will be 900E.

Selcet“Repair Mode”，select the QCN file which we have saved in the folder SW.

Click “STRAT” button,the tool will to  upgrade form  modem

（2）To update AP failed

At first, select“Repair Mode”and select the QCN file which we have saved in the folder SW, then click “STRAT” button.

Holding down down-key of volume, then connect PC. Now the port will be Android Phone.

The tool will to upgrade form android.