How to update software from sdcard

# Preparation

- Make sure the unused memory of the SD card is more than 512M bytes.

- Copy the update file(such as “Croma_20130715.zip”) to root directory of the SD card, then insert the SD card to the phone and install the battery.

- Make sure that the battery has enough energy. Such as the percentage of battery is more than 30 percent.

# Begin to update

- Enter recovery mode

  - Press the “volume up” key and “power on” key at the same time until the phone enter recovery mode, which shown as picture-1.

- Select file to update

  - Press the “volume up/down” key to select “apply update from sdcard”. Shown as picture-1

  - Press the “power on” key to confirm the selection, the phone will enter another interface. Shown as picture-2.

  - Press “volume down” to select the update file (here is “Croma_20130715.zip”).

  - Press “power on” to confirm, and the phone begin to update.

  - The process of update will last more than 1 minute, please wait more patiently. The phone will reboot 3 times automatically during the process. Shown as picture-3, picture-4

# Check whether the update is successful

- After several steps above, the phone is already power on. Please check the last 4 numbers of build number of the phone is consistent to last 4 numbers of the update file(here is 0715).

If they are the same, update successfully. Or, it updates failed, please check which step is wrong.

# Contact us

Picture-1

Picture-2

Picture-3

Picture-4