USB Flash download quick start guide

# Platform Supported

  - 213x1 VENUSRAY board

  - 2153 TORPEDORAY D0 board

# Build the USB flash utility driver

The USB flash utility driver is a standalone component which can be added into tool build chain by enabling or defining the following flag in your platform file, USB_FLASH_DOWNLOAD_DRIVER_BUILD=TRUE

Then run “make” command. Make sure the file “usb_flash_drv.img” is generated in the \tool folder.

The source files required are labeled in the USB_Flash_Download_Driver label in Perforce. Just simply sync to this label if your sources does not have it or the update to the latest.

# Development switch setting

If the development board is used, make sure the switch is set to USB download mode (SW68 pin 1 is at off position for 2153 board and SW4 pin 1 is at off position for Venus board).

# Install USB PC driver

You will need USB PC driver installed on your PC and if you do not know where to find the PC driver, you can copy from the following shared the server if you have the access to it:

\\wc-sjca-m001\wm5\eliang\USB_Driver

  - Copy the PC driver to your local hard drive

  - Connect the USB cable between target device and PC

  - Power on the device/board and the PC will detect the new device found and ask for the driver to install

  - Select the PC driver from where you copy at the first step

  - Reset the board and you should be Broadcom USB Wireless handset appear in the Modems in the device manager as shown below:

# Flashing batch file

There are 2 separated batch file created for USB download which goes in pair with their UART download batch file. The file name follows the convention of UART batch file with additional of “_USB” at the end of the file name.

For example:

For Venus board, spansion.bat is the one for UART and spansion_usb.bat is the one for USB.  The following is to flash all the images to venusRay board with NOR DDR memory card:

Spansion_usb.bat all NOR_DDR_LE

# Known issue and Workaround

Due to an unreliable USB PC driver, the flashing may be failed in the middle (com port error) some times. Press the reset button and hold for 1 or 2 seconds till the Broadcom USB wireless Handset device disappear in the device manager, in which case the OS will reload the USB stack.